# Concrete Pump Line Water Cleanout JHA

Worked instructional example

# Work and worksite

A specialist pump operator and assigned helpers will carry out routine manufacturer-permitted water cleanout of a ground-level concrete delivery pipeline after a completed building pour. A controlled receiving arrangement contains the discharge; the actual equipment procedure governs all pressure and isolation actions. This fictional learning example describes planned controls; it records no actual inspection or work authorization.

The pump, line, fittings, supports, cleaning method and receiving equipment must be compatible under current manufacturer instructions.

This is a planning example for a known, unobstructed line; it provides no valve sequence, pressure setting or cleaning-device recipe.

It covers part of concrete pump cleanout. Compressed-air cleaning, blockage removal, high-rise risers, hopper entry and equipment repair are excluded.

## Questions before work

Does the actual pump/line procedure permit this water method with the available receiving equipment?

Who controls every pressure source and verifies the safe condition before a connection is touched?

How will the discharge exclusion area, containment capacity and operator-helper communication be maintained?

# Task Hazard Control

## 1 Release the routine cleanout method

Hazard: An unsuitable cleaning arrangement can separate fittings or discharge material unexpectedly.

Control: Pump operator checks the current manufacturer procedure, line condition and compatible cleanout/receiving components. Confirm the operation is routine cleaning, not suspected blockage removal; stop if equipment or method suitability is unresolved.

Verification: Operator and site lead confirm the written method and scope before setup.

Reasoning: Water cleaning still needs an equipment-specific pressure and discharge plan.

## 2 Establish the receiving boundary

Hazard: Concrete, water or a cleaning device can strike a person or overflow into a travel route.

Control: Site lead provides the prescribed receiving/catching arrangement and adequate contained capacity. Close the discharge and line-of-fire area to workers and other trades; select protected observation and helper positions outside potential discharge paths.

Verification: Lead checks receiving equipment, capacity and boundaries with the operator before connection work.

Reasoning: A water method does not remove the hazard at the line outlet.

## 3 Verify isolation for setup

Hazard: Residual or reintroduced pressure can eject a coupling or drive unexpected equipment movement.

Control: Operator controls all identified energy/pressure sources using the actual procedure. Verify the specified safe condition before any connection or guard is disturbed; keep helpers clear and do not treat a single gauge indication as complete verification.

Verification: Operator confirms the verified condition and authority to approach each connection.

Reasoning: A stopped pump and a safely depressurized line are different states.

# Complete the work and hand over

## 4 Assemble and check the arrangement

Hazard: Wrong connectors, insecure supports or bypassed restraint can let the line or outlet move.

Control: Assigned personnel assemble only the manufacturer-permitted components after isolation verification. Check connection compatibility, locks, line supports, restraint and discharge capture against instructions; do not improvise caps or fittings.

Verification: Operator checks the complete arrangement and clears helpers before initiating cleaning.

Reasoning: Compatible parts and a contained outlet must function as one system.

## 5 Conduct cleaning under operator control

Hazard: Unexpected movement, interrupted discharge or changed pressure can expose helpers to hose whip or material release.

Control: Operator follows the approved water-cleanout procedure while maintaining communication and the exclusion area. Observe from designated positions; stop and return to the prescribed safe state for abnormal response. Do not use compressed air to force a stalled cleanout.

Verification: Operator and boundary attendant monitor line response, receiving capacity and continued separation.

Reasoning: An abnormal response ends this routine scenario; it does not authorize blockage clearing.

## 6 Verify safe closeout and hand over

Hazard: Opening a connection too early can release retained pressure; contaminated residue can expose cleanup workers.

Control: Use the actual shutdown, pressure-release and verification procedure before disassembly. Keep discharge restrictions until the safe condition is established, contain residue and hand remaining defects or cleanup restrictions to the site lead.

Verification: Operator records safe-state verification and unresolved defects; lead accepts the contained waste area.

Reasoning: Closeout must verify the line’s condition before another person approaches.

## Pause and reassess

No discharge, unexpected movement or another abnormal line response.

Receiving equipment, capacity, connection security or exclusion boundary becomes unsuitable.

A pressure source, verification method or communication arrangement cannot be controlled.

# Improve the controls

Needs improvement: Water is safer, so flush it out.

More useful: Operator first verifies a permitted method, compatible components, controlled receiving equipment and an exclusion area; cleaning remains under model-specific instructions.

Needs improvement: The motor is stopped; undo the coupling.

More useful: Operator isolates identified sources and verifies the prescribed safe pressure condition before any helper touches the connection.

## Decisions that change the analysis

May the operator use air when water cleanout stalls?

This example excludes compressed-air cleaning and blockage removal. A stalled or abnormal response requires stopping, establishing the prescribed safe state and obtaining specialist review under the applicable procedure; it does not justify an improvised method.

What must the crew record for actual use?

Record pump and line identity, the procedure revision, compatible cleanout/receiving components, discharge boundaries, responsible operator/helpers, actual safe-state findings, abnormal observations and waste handover. This JHA supplies no equipment-operating sequence.

## Training needs

Model-specific water-cleanout, pressure verification, isolation and emergency instruction for the operator and assigned helpers.

Crew instruction in hose/line-of-fire boundaries, controlled receiving arrangements and wet-concrete exposure response.

Confirm applicable qualifications and actual training evidence for the people assigned. Related toolbox talks support discussion; they do not establish competence.

## References and adaptation

OSHA 1926.702 Concrete equipment and tools
https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.702

WorkSafe Queensland Concrete pump hose-whip alert
https://www.worksafe.qld.gov.au/news-and-events/alerts/workplace-health-and-safety-alerts/2011/hose-whip-on-concrete-pumps

US EPA Concrete washout best management practice
https://www.epa.gov/sites/default/files/2015-11/documents/concretewashout_0.pdf

References reviewed October 9, 2026. Use actual equipment and product instructions, project design and site requirements. This instructional example supplies no actual inspection results, signatures or work approval.