# Glass Rack and A Frame Handling JHA

Worked instructional example

# Work and worksite

A glazing crew will receive and stage glass on a specified transport rack and transfer selected panes to a secured shop-floor A frame using the planned handling method. This fictional learning example records planned controls; it supplies no actual inspections or work authorization.

Rack capacity, restraint, loading balance and handling points come from the actual system instructions.

The staging floor and movement route must support the rack and handling equipment.

This excludes hoisting an unrated rack, trailer access and installing glass in an opening.

## Questions before work

Which rack, panel load and restraint configuration are permitted?

What supports the rack against sliding or overturning?

How will restraint be changed while selected panes remain supported?

# Task Hazard Control

## 1 Release the staging location

Hazard: An uneven or overloaded floor can allow a rack to tip or slide.

Control: Lead selects the released support location and route for the loaded rack, preserving access and separation from moving equipment.

Verification: Check floor support and clearances for the actual rack and load.

Reasoning: An empty rack’s stable appearance does not establish loaded stability.

## 2 Inspect the rack and delivered glass

Hazard: Damaged stops, restraints or glass can cause unexpected movement or breakage.

Control: Inspect the rack, load restraint and panel condition under the system procedure. Identify defects and keep affected loads secured for disposition.

Verification: Lead checks component condition and panel identification before transfer.

Reasoning: Packaging must remain controlled while defects are assessed.

## 3 Prepare the receiving A frame

Hazard: A poorly supported or overloaded frame can overturn during transfer.

Control: Set and secure the receiving frame by its instructions and confirm the permitted load distribution. Prepare padding, stops and restraints before moving a pane.

Verification: Compare the frame setup and intended load with its instructions.

Reasoning: The receiving frame must be ready before delivery restraints change.

# Complete the work and hand over

## 4 Release only the pane being handled

Hazard: Other panes can fall when common restraint is removed.

Control: Use the specified sequence to keep remaining glass restrained and the selected pane continuously supported. Position workers outside the pane’s fall and pinch paths.

Verification: Lead confirms support and remaining restraint before each release.

Reasoning: A common strap may protect more than the selected pane.

## 5 Transfer and restrain the pane

Hazard: Glass can cut, crush or strain workers during movement.

Control: Use the planned rated handling device or assessed team method and suitable contact protection. Place and restrain the pane on the receiving frame before releasing handling support.

Verification: Lead checks route clearance and receiving restraint for each transfer.

Reasoning: Restraint must follow the pane throughout the transfer.

## 6 Secure the staged load and hand over

Hazard: An open restraint or unidentified damaged pane can injure the next crew.

Control: Check all remaining loads, mark defects and preserve the rack movement restrictions. Communicate load configuration and approved handling method.

Verification: Lead completes a rack-by-rack closeout check.

Reasoning: A partially unloaded rack may have a different balance.

## Pause and reassess

Rack damage, unknown capacity or unstable support appears.

Load balance or common restraint differs from the plan.

Glass damage or an unplanned handling method is proposed.

# Improve the controls

Needs improvement: Be careful around the equipment.

More useful: Set and secure the receiving frame by its instructions and confirm the permitted load distribution. Prepare padding, stops and restraints before moving a pane.

Needs improvement: Check the area before starting.

More useful: Lead selects the released support location and route for the loaded rack, preserving access and separation from moving equipment.

## Decisions that change the analysis

Can transport packaging be assumed suitable for repeated site storage?

No. Confirm its intended use, support, restraint and load limits. Use a suitable staging system when the transport packaging cannot meet the planned storage conditions.

What must be recorded before this example is used on a job?

Replace the scenario with the actual work, identify additional hazards with the crew, and record the applicable instructions, responsible people, inspection findings, unresolved actions and required authorization. A planned verification is not an inspection result.

## Training needs

Actual rack restraint, loading balance and pane-transfer method.

Glass damage recognition and safe hand and body positions.

Confirm applicable qualifications and actual training evidence for the people assigned. Related toolbox talks support discussion; they do not establish competence.

## References and adaptation

OSHA 1926.300 Hand and power tools
https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.300

OSHA 1926.21 Safety training and education
https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.21

References reviewed October 9, 2026. Use actual equipment and product instructions, project design and site requirements. This instructional example supplies no actual inspection results, signatures or work approval.