# Post Frame First Bay Erection JHA

Worked instructional example

# Work and worksite

A post-frame building crew will establish the first stable bay using identified engineered wood columns on released surface-mounted concrete-pier connections, followed by the specified girts and temporary braces. Roof-truss work begins under a separate analysis after the bay handover. This fictional learning example describes planned controls; it records no actual inspection or work authorization.

Current erection documents define pier readiness, column bases, member orientation, girt connections and the bay’s temporary stability sequence.

Crane handling and protected connection access are arranged for the actual columns and bay configuration.

Embedded-post excavation, foundation construction, roof-truss erection, cladding and whole-building structural design are excluded.

## Questions before work

Which foundation records and base details release each column position?

How will the first columns remain stable before the bay’s girts and braces are complete?

Who verifies the bay before crane support or later construction restrictions change?

# Task Hazard Control

## 1 Release the firstbay sequence

Hazard: An incomplete foundation or unsupported first column can cause progressive movement or collapse.

Control: Erection lead checks pier and base release, column identity and the designed first-bay sequence. Stage the specified braces and anchorage before any lift; hold the operation where the stability plan or foundation connection is unresolved.

Verification: Lead compares both column positions and stability measures with current documents.

Reasoning: The first bay has no completed neighboring frame to assume as restraint.

## 2 Prepare handling and protected access

Hazard: An unsuitable lifting attachment or unprotected connection position can expose workers to a falling column or fall.

Control: Use the approved column handling method with suitable rated equipment and applicable qualified lift roles. Establish connection platforms or the selected fall-protection arrangement, rescue and the excluded area below; preserve clear movement and escape routes.

Verification: Handling lead verifies attachments and receiver access before pickup.

Reasoning: Large wood columns need specific handling beyond ordinary stud-wall assembly.

## 3 Set and restrain the first column

Hazard: A tall column can rotate, trap a receiver or fall when the crane is released.

Control: Guide from planned positions outside swing and pinch paths and seat onto the identified base. Keep crane support while the specified base connection and initial temporary braces are completed; obtain the designated lead’s deliberate release before unrigging.

Verification: Erection lead checks seating, connection and restraint for the first column.

Reasoning: Foundation attachment alone may not supply erection-stage lateral stability.

# Complete the work and hand over

## 4 Set the next column and connect girts

Hazard: An incompletely connected bay or unsupported girt can move or drop onto workers.

Control: Follow the released sequence for the next column and girts using controlled support and protected access. Maintain each member’s support until required connections and restraints are checked; do not use a loose girt as a walkway or substitute the next member for a missing brace.

Verification: Lead checks each connection and support transition at the planned hold points.

Reasoning: The bay becomes stable through a sequence, not simply by adding more timber.

## 5 Verify bay restraint and alignment

Hazard: Adjusting or removing a brace can release forces into an incomplete frame.

Control: Confirm the specified bay bracing, anchor conditions and alignment through the erection plan’s hold point. Make adjustments only by the released method while maintaining stability; stop for unexpected base movement, member response or an unavailable acceptance criterion.

Verification: Designated lead records actual alignment and restraint findings before bay release.

Reasoning: A plumb-looking column does not prove that the bay’s restraint is complete.

## 6 Hand over before roof work

Hazard: Later roof crews or material loads can disturb braces needed until permanent elements are complete.

Control: Identify and protect temporary braces, secure loose members and transfer bay status, loading limits and removal authority. The separate truss operation starts only after its receiving-support release; retain bay restraint until its design-defined release.

Verification: Receiving roof lead confirms support readiness and continuing bay restrictions.

Reasoning: A first-bay handover is one stage of the post-frame building.

## Pause and reassess

Pier, base, member orientation or the first-bay sequence differs from current documents.

A column, brace or base moves unexpectedly or protected access is lost.

An adjustment, construction load or brace removal lacks the required release.

# Improve the controls

Needs improvement: Work carefully and wear PPE.

More useful: Guide from planned positions outside swing and pinch paths and seat onto the identified base. Keep crane support while the specified base connection and initial temporary braces are completed; obtain the designated lead’s deliberate release before unrigging.

Needs improvement: Check that everything is ready.

More useful: Erection lead checks pier and base release, column identity and the designed first-bay sequence. Stage the specified braces and anchorage before any lift; hold the operation where the stability plan or foundation connection is unresolved.

## Decisions that change the analysis

Can the roof-truss bracing instructions replace the first-bay plan?

No. Truss guidance addresses roof assemblies after receiving support is ready. Columns, bases and girts require the post-frame building’s own temporary stability sequence before roof erection starts.

What must the crew record for actual use?

Identify the actual equipment and work boundaries, responsible people, applicable criteria, inspection findings and open actions. Resolve changes with affected crews and obtain required project authorization before work.

## Training needs

The actual post-frame first-bay sequence, base release, brace installation and adjustment/removal authority.

Applicable lifting qualifications, communication and fall-protection/rescue instruction for column and girt work.

Confirm applicable qualifications and actual training evidence for the people assigned. Related toolbox talks support discussion; they do not establish competence.

## References and adaptation

NFBA Post-frame technical resources
https://www.nfba.org/aws/NFBA/pt/sp/tech-resources

Lester Buildings Post-frame jobsite safety and temporary bracing
https://www.lesterbuildings.com/blogs/pole-barn-jobsite-safety/

OSHA 1926.501 Duty to have fall protection
https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.501

References reviewed October 9, 2026. Use actual equipment and product instructions, project design and site requirements. This original draft supplies no fabricated inspection, signature or approval.