# Vacuum Glass Lifter Operation JHA

Worked instructional example

# Work and worksite

A glazing crew will move a specified glass panel from a secured rack to a receiving support with a vacuum lifting device and an approved hoisting arrangement at ground level. This fictional learning example records planned controls; it supplies no actual inspections or work authorization.

Panel weight, surface, geometry and orientation must suit the actual lifter configuration and instructions.

The receiving support and controlled travel area are ready before attachment.

This excludes elevated glazing, damaged glass and treating a hand suction cup as a rated hoisting device.

## Questions before work

What confirms panel and lifter compatibility in the intended orientation?

Which vacuum checks and alarm responses does this unit require?

How will the panel be supported before vacuum is released?

# Task Hazard Control

## 1 Identify the panel and movement

Hazard: Unknown weight or unsuitable glass can exceed device limits or lose attachment.

Control: Glazing lead confirms panel identification, weight, condition and surface compatibility with the lifter’s configuration and planned movement. Hold unresolved capacity or suitability.

Verification: Compare the actual panel with lifting documents and the unit instructions.

Reasoning: A nominal capacity alone may not apply to every orientation or surface.

## 2 Prepare receiving support and the route

Hazard: A suspended panel can strike people or arrive at an unstable support.

Control: Set the receiving support and separate the travel, swing and potential fall zones. Agree signals and safe guide positions.

Verification: Lead checks the route and receiving support before attachment.

Reasoning: The receiving arrangement must exist before the panel moves.

## 3 Inspect and attach the lifter

Hazard: Damaged pads, contamination or failed vacuum controls can drop glass.

Control: Use the manufacturer’s inspection, pad preparation, attachment and specified safe function-check procedure. Verify required indicators and alarms; remove a defective lifter from service.

Verification: Authorized operator records the unit-specific checks before the lift.

Reasoning: Pad contact does not prove a reliable vacuum attachment.

# Complete the work and hand over

## 4 Move the panel under control

Hazard: A vacuum loss, swing or rotation can crush or cut workers.

Control: Use the planned hoist and lifter method, maintain required indicators and keep people out of the panel’s fall and pinch paths. Follow the unit’s response to an alarm; never stand beneath to catch glass.

Verification: Operator monitors attachment status; lift lead maintains communication and boundary.

Reasoning: An alarm requires the prescribed response rather than a faster lift.

## 5 Support the panel and release vacuum

Hazard: Early release can drop or overturn the panel.

Control: Land on the prepared support and secure the panel by the planned receiving method. Release vacuum only after the designated lead confirms independent support.

Verification: Lead verifies support and restraint before authorizing release.

Reasoning: Independent support ends reliance on the lifting device.

## 6 Secure the device and hand over

Hazard: An unsecured panel or damaged device can expose the next user.

Control: Confirm the panel’s storage or installation restraint, inspect the lifter after use and isolate defects. Communicate actual panel and equipment status.

Verification: Glazing lead checks restraint and the device’s closeout condition.

Reasoning: The lift ends with both glass and equipment in a controlled state.

## Pause and reassess

Panel surface, weight or orientation differs.

Vacuum indicators, alarms or attachment behavior change.

Wind, travel clearance or receiving support changes.

# Improve the controls

Needs improvement: Be careful around the equipment.

More useful: Use the manufacturer’s inspection, pad preparation, attachment and specified safe function-check procedure. Verify required indicators and alarms; remove a defective lifter from service.

Needs improvement: Check the area before starting.

More useful: Glazing lead confirms panel identification, weight, condition and surface compatibility with the lifter’s configuration and planned movement. Hold unresolved capacity or suitability.

## Decisions that change the analysis

Can a hand suction cup replace a vacuum lifter?

Only equipment specifically suitable for the intended load and lifting arrangement can be used. A hand cup’s appearance or label does not establish suitability for hoisting; reassess against its instructions and the lift design.

What must be recorded before this example is used on a job?

Replace the scenario with the actual work, identify additional hazards with the crew, and record the applicable instructions, responsible people, inspection findings, unresolved actions and required authorization. A planned verification is not an inspection result.

## Training needs

Actual vacuum lifter setup, checks, alarms and prescribed response.

Applicable hoist, rigging and signaling roles and qualifications.

Confirm applicable qualifications and actual training evidence for the people assigned. Related toolbox talks support discussion; they do not establish competence.

## References and adaptation

OSHA 1926.300 Hand and power tools
https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.300

OSHA 1926.21 Safety training and education
https://www.osha.gov/laws-regs/regulations/standardnumber/1926/1926.21

References reviewed October 9, 2026. Use actual equipment and product instructions, project design and site requirements. This instructional example supplies no actual inspection results, signatures or work approval.