TOOLBOX TALK 182  •  TOOLBOX DISCUSSION

# Silica Cleanup: Keep Settled Dust from Becoming Exposure

Todd Jerome Jenkins  |  v1.0  |  October 12, 2026  |  Discussion + field verification

# Why this matters

Cutting may be finished while exposure is still possible. Sweeping, emptying a collector or disturbing dried residue can put settled silica dust back into the air.

Objective: Identify the approved cleanup method and verify its equipment and waste-handling steps.

# Key hazards or failure points

Dry sweeping; compressed-air cleanup; leaking collection equipment; dust released during emptying; exposure of nearby crews.

# Practical controls

Use the employer’s silica plan for cleanup and waste handling. Have the competent person resolve an unlisted task before starting.

Use planned wet methods, HEPA-filtered vacuuming or another approved method that minimizes exposure; prevent additional slip or electrical hazards.

Control access and include collector emptying, filter maintenance and required respiratory protection in the plan.

# Verify It in the Field

The competent person matches the residue and cleanup task to the written plan, including nearby-worker exposure.

The operator checks the specified vacuum, filters, seals and prescribed function indicators against manufacturer instructions; a running motor is insufficient evidence.

Confirm the waste-transfer method, containers and receiving area before opening collection equipment. Record actual findings, responsible person and time.

# Stop and reassess

Hold for an unknown residue, missing method, damaged collector or uncontrolled dust. Do not improvise dry sweeping or compressed-air cleaning. Correct, recheck and rebrief before restarting.

# Ask the crew

1. What could release dust during cleanup?

2. What proves this equipment fits the method?

3. Who resolves an exception or changed condition?

Use this HOP tool — Procedure use and adherence: Choose the cleanup step most likely to release dust. Compare the current instruction with the residue, equipment and waste route in front of you. Name a mismatch that would require a hold, and identify who can revise and approve the method.

Sources: OSHA 1926.1153(f)–(g); OSHA construction silica FAQs; DOE Vol. 2, pp. 20–21. HOP concepts adapted from DOE-HDBK-1028-2009, Vol. 2 (June 2009). No claim to original U.S. Government work; no government endorsement implied.

Free next step: JHA/JSA planning and field-verification worksheet

Expanded guidance and separate forms: toddjeromejenkins.com/toolbox-talks/silica-dust-cleanup