TOOLBOX TALK 183  •  TOOLBOX DISCUSSION

# Face Shields and Respirators: When Protection Interferes

Todd Jerome Jenkins  |  v1.0  |  October 12, 2026  |  Discussion + field verification

# Why this matters

A face shield can contact a respirator or displace eyewear when the worker turns or bends. Study the actual combination before exposure, especially after changing a component.

Objective: Verify that the selected shield, eyewear and respirator work together without disrupting required protection.

# Key hazards or failure points

Shield contacting a respirator; displaced eyewear; fogging; obscured view; damaged attachments; disturbed facepiece seals.

# Practical controls

Use the respiratory program and task assessment to select compatible respirator, shield and primary eye protection; consult actual product instructions.

Resolve interference through approved selection or work-method changes. Do not lift required protection, break the respirator seal or improvise attachments.

Keep engineering controls effective. Have the respiratory-program administrator resolve compatibility or fit concerns before the combination is used.

# Verify It in the Field

The lead and wearer identify actual equipment models and compare the complete configuration with program selection and manufacturer instructions.

In a safe area, don the complete set and use normal task head movements; study shield contact, displacement and the sightline.

For a tight-fitting respirator, the wearer performs the prescribed user seal check. Confirm required fit-testing arrangements separately; peer observation cannot replace them.

# Stop and reassess

Leave exposure safely for shifting protection, obscured vision or seal leakage. Hold the task until the combination is corrected and rechecked; do not remove a required component to continue.

# Ask the crew

1. Could the shield touch the respirator during this task?

2. What changes when we look down?

3. Who resolves an incompatible combination?

Use this HOP tool — Peer checking: In a safe area, the wearer and a familiar coworker identify the planned shield and respirator combination. Agree the approved donning sequence, then observe normal task movements for contact or displacement. Record a concern and resolve it through the program before exposure.

Sources: OSHA 1926.102; OSHA 1926.95(c); OSHA 1910.134(g)(1); DOE Vol. 2, pp. 41–43. HOP concepts adapted from DOE-HDBK-1028-2009, Vol. 2 (June 2009). No claim to original U.S. Government work; no government endorsement implied.

Free next step: JHA/JSA planning and field-verification worksheet

Expanded guidance and separate forms: toddjeromejenkins.com/toolbox-talks/face-shield-respirator-interference