TOOLBOX TALK 95  •  CREW DISCUSSION

# Rebar Cages: Verify Support Before Release

Todd Jerome Jenkins  |  v1.0  |  Reviewed October 9, 2026  |  Discussion + field checks

# Why this matters

A tied cage is not automatically stable through assembly, lifting and placement. Each change of support needs a defined arrangement before people enter its possible fall or movement path.

Objective: Identify what supports the cage at each stage and verify the receiving restraint before crane release.

# Key hazards or failure points

Unstable partial cage; unsuitable lift points; deformation; overturned cage; trapping at the base; disturbed restraint; projecting steel.

# Practical controls

Use the cage-specific assembly, lifting and temporary-support information. Confirm the supported assembly location and receiving area before work; do not assume tie wire establishes lifting capacity.

Keep people clear of the moving cage and its potential collapse path. Use the planned access and load-control method; this talk does not authorize climbing reinforcement.

Install the specified receiving restraint before releasing the crane. Preserve temporary supports and communicate who may approve changes or their removal.

# Verify It in the Field

Match the cage identity and current arrangement to the assembly and lift documents. Responsible lift personnel verify the lifting points and gear; appearance alone cannot establish capacity.

From safe access, check actual receiving support and required restraint against the plan. Confirm who checks the installed arrangement and communicates release; do not test stability by unhooking.

Record cage/location, support source, actual findings, checker and restrictions. Confirm impalement protection and access for the next crew without entering an unstable cage.

# Stop and reassess

Hold work for missing support evidence, changed shape, unexpected movement, damaged restraint or an unsuitable receiving area. Keep clear; never try to catch or manually brace a falling cage. Obtain technical correction and recheck before continuing.

# Ask the crew

1. What supports the cage during this next step?

2. What must be verified before the crane releases it?

3. Who must approve changing a restraint?

Use the task plan and required specialist decisions before exposure.

Sources: OSHA 1926.703(d)(1); 1926.701(b); actual cage and lift documents.

Free next step: JHA/JSA planning & field-verification worksheet

Expanded guidance and separate forms: toddjeromejenkins.com/toolbox-talks/rebar-cage-stability